[Company Name] 	MSDS – [Product Name] 	Page 1 of 3

# MATERIAL SAFETY DATA SHEET

SECTION I – IDENTIFICATION

| Product identifier |  |
| --- | --- |
| Product Use Soil Conditioner, Soil Remediation, Manufacturing Ingredient, Cement Replacement |  |
| Provider |  |
| Address |  |
| City | State |
| Zip code | Emergency Telephone |
| Date prepared |  |
| Prepared by | Phone |

SECTION II – HAZARD IDENTIFICATION

Hazard Classification of Wakefield Biochar Soil Conditioner: - Flammable Wood – Category 5 - Aspiration Hazard – Category 4 - Carcinogenicity – Category 5 - Specific Target Organ Toxicity (Repeated Exposure) – Category 5 - Specific Target Organ Toxicity (Single Exposure) – Category 5 - Skin Irritation – Category 5 - Eye Irritation – Category 5 - Chronic Aquatic Toxicity – Category 5 Where the category numbers refer to how severe the hazard is with 1 being the most hazardous and 5 being the least hazardous.

SECTION III – COMPOSITION/INFORMATION ON INGREDIENTS

| Ingredients | % | ACGIH TLV-TWA | ACGIH STEL | OSHA PEL |
| --- | --- | --- | --- | --- |
| % Carbon |  | 10 mg/m3 | 15 mg/m3 | 5 mg/m3 |
| % Ash |  | Not listed |  |  |
| Moisture Content |  | Not listed |  |  |

SECTION IV – FIRST-AID MEASURES

OVER EXPOSURE

[Company Name] 	MSDS – [Product Name]

Page 2 of 3 Exposure to biochar should not present a health hazard. Carbon dust in excessive amounts when handling may produce an allergenic response in sensitive individuals.

| Skin Contact | Prolonged contact with skin may result in slight irritation/redness in sensitive individuals. Should be remediated by rinsing thoroughly. Seek medical attention if persists. |
| --- | --- |
| Eye Contact | May irritate the eyes and cause watering and redness in sensitive individuals. Rinse thoroughly with plenty of water to remove foreign bodies. Seek medical attention if persists. |
| Inhalation | Inhalation of dust may irritate nose, throat, and lungs. May aggravate pre-existing conditions. Remove affected person to fresh air. Seek medical attention if persists. |

SECTION V – FIRE-FIGHTING MEASURES

| Flammable _X_yes ____no | If yes, under what conditions? Near direct flame or when exposed to extreme temperatures (>700°C). |  |
| --- | --- | --- |
| Means of Extinction Water, carbon dioxide, dry chemical, or chemical foam |  |  |
| Autoignition Temperature >700 °C | Explosion Data – Sensitivity to Impact NA | Explosion Data – Sensitivity to Static Discharge Fine dust particle above 0.140 g/l in the presence of an ignition source presents dust explosion hazard. |
| NFPA Use water to cool exposed containers and remove from path of fire if safe to do so. Wear self-contained breathing apparatus with full face mask and protective clothing. |  |  |

SECTION VI – ACCIDENTAL RELEASE MEASURES

| Leak and Spill Procedures | Ensure area is well ventilated. Vacuum or sweep material and dispose of in suitable container according to local disposal guidelines. |
| --- | --- |
| Cautions | Avoid creating dust and be cautious of smoldering. May smolder if exposed to heat over 550 degrees Celsius. |

SECTION VII – HANDLING AND STORAGE

| Handling Procedures | Avoid dust release during storage or handling. |
| --- | --- |
| Storage Requirements | Store below 80C and keep covered to limit dust. Do not store with oxidizing agents. |

SECTION VIII – EXPOSURE CONTROLS/PERSONAL PROTECTION

Gloves: gloves are recommended to reduce skin contact, particularly in sensitive individuals.

Respirator: If dust is excessive during handling, Wear a NIOSH/OSHA approved particle respirator (p100).

Eye: Wear safety glasses with side shields.

Ventilation: When possible exhaust ventilation should be used.

SECTION IX – PHYSICAL AND CHEMICAL PROPERTIES

| Physical State Solid, particle size 0.1 – 2mm | Appearance Black carbonaceous material | Odor none |
| --- | --- | --- |

[Company Name] 	MSDS – [Product Name]	Page 3 of 3

| pH 8 – 9 | Boiling Point/Range NA | Melting/Freezing Point NA |
| --- | --- | --- |
| Vapor Pressure (mmHg) NA | Solubility INSOLUBLE IN WATER |  |

SECTION X – STABILITY AND REACTIVITY

| Chemical Stability _X__yes _____no | If no, under which conditions? |
| --- | --- |
| Incompatibility with Other Substances __X_yes ____no | If yes, which ones? Strong Oxidizers and open flames |
| Hazardous Decomposition Products Combustion of product produces carbon monoxide and hydrocarbons and organic acids. |  |

SECTION XI – TOXICOLOGICAL INFORMATION

DATA NOT AVAILABLE

SECTION XII – ECOLOGICAL INFORMATION

DATA NOT AVAILABLE

SECTION XIII – DISPOSAL CONSIDERATIONS

Waste Disposal

Sweep or vacuum product. Incinerate or dispose of in accordance with all federal, state, and local regulations.

SECTION XIV – TRANSPORT INFORMATION

Always ensure load is covered to prevent dust loss. When tipping or unloading ensure proper PPE is worn. Biochar is hygroscopic and can absorb liquids and gases.

SECTION XV – REGULATORY INFORMATION

No additional information

SECTION XVI – OTHER INFORMATION

Additional Information:

The above information is your notice as to the Right-to-Know listings of the stated products. The information contained herein is based on data available at this time and is believed to be accurate to the best of our knowledge. As of November 3, 2016, this is the most up to date information [Company Name] can give. [Company Name] will do everything in its power to ensure that updated versions of this MSDS is distributed to the appropriate customers but ultimately the responsibility of obtaining up to date SDS belongs to the user. Since information contained herein may be applied under conditions beyond our control and with which we may be unfamiliar, no responsibility is assumed for the results or damage of its use.